DAY - 8

For each RTL write all sections: Overview, Key Features, Interface Table, Microarchitecture, FSM Specification, Timing, Corner Cases, RTL Code.

## MODULE 1: Sequence Detector (1011 Overlapping)

### 1.1 Overview

This module is a synchronous finite state machine designed to detect the serial binary pattern "1011" from a continuous input stream. It is designed to handle overlapping sequences without dropping detections.

### 1.2 Key Features

- Detects the target sequence "1011" in a continuous serial stream.

- Supports overlapping pattern detection.

- Data input is gated by a valid signal to pause detection without losing state.

- Synchronous active-low reset for safe initialization.

### 1.3 Interface Table

| Port Name | Direction | Width | Clock Domain | Active Level | Description |
| --- | --- | --- | --- | --- | --- |
| clk | Input | 1 | N/A | Rising Edge | Main system clock. All state transitions occur on the positive edge. |
| rst_n | Input | 1 | Asynchronous | Active-Low | Asynchronous reset. Forces the FSM to the IDLE state immediately. |
| i_valid | Input | 1 | clk | Active-High | Data Valid flag. When HIGH, indicates i_bit contains valid data to sample. |
| i_bit | Input | 1 | clk | N/A | The serial data input stream. Sampled only when i_valid is HIGH. |
| o_match | Output | 1 | clk | Active-High | Match flag. Pulses HIGH for one clock cycle when the sequence is detected. |

### 1.4 Microarchitecture & FSM Specification

State Encoding Table:

| State Name | Encoding ([2:0]) | Description |
| --- | --- | --- |
| S_IDLE | 3'b000 | Reset state. Waiting for the first '1'. |
| S_1 | 3'b001 | Detected "1". |
| S_10 | 3'b010 | Detected "10". |
| S_101 | 3'b011 | Detected "101". |
| S_1011 | 3'b100 | Detected "1011",Drives o_match = 1. |

Transition Table:

| Current State | Condition (i_bit) | Next State | Output (o_match) |
| --- | --- | --- | --- |
| S_IDLE | 0 | S_IDLE | 0 |
| S_IDLE | 1 | S_1 | 0 |
| S_1 | 0 | S_10 | 0 |
| S_1 | 1 | S_1 | 0 |
| S_10 | 0 | S_IDLE | 0 |
| S_10 | 1 | S_101 | 0 |
| S_101 | 0 | S_10 | 0 |
| S_101 | 1 | S_1011 | 0 |
| S_1011 | 0 | S_10 | 1 |
| S_1011 | 1 | S_1 | 1 |

ASCII State Diagram:

### 1.5 Timing Diagrams

- i_bit must be stable before the rising edge of clk when i_valid is HIGH.

- o_match asserts synchronously on the clock edge immediately following the transition into the S_1011 state.

### 1.6 Corner Cases

| Scenario | Expected Behaviour | How it is Handled in RTL |
| --- | --- | --- |
| Overlapping Sequence (1011011) | Must assert o_match twice. It must not reset to IDLE after the first match. | Transition from S_1011 with i_bit=0 routes back to S_10, retaining the start of the next sequence. |
| Reset Mid-Sequence | If rst_n asserts while in S_101, state must wipe instantly without pulsing o_match. | State register uses an asynchronous active-low reset prioritizing IDLE. |
| Invalid Data Toggle | If i_bit toggles while i_valid is LOW, the state machine must not advance. | FSM state transitions are strictly gated inside an if(i_valid) block. |

### 1.7 RTL Code (Revised)

module seq_detector (

input  logic clk,

input  logic rst_n,

input  logic i_valid,

input  logic i_bit,

output logic o_match

);

// State Encoding

typedef enum logic [2:0] {

S_IDLE = 3'b000,

S_1    = 3'b001,

S_10   = 3'b010,

S_101  = 3'b011,

S_1011 = 3'b100

} state_t;

state_t current_state, next_state;

// Sequential Block: State transitions with active-low async reset

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

current_state <= S_IDLE;

end else if (i_valid) begin

current_state <= next_state;

end

end

// Combinational Block: Next State logic handling overlaps

always_comb begin

next_state = current_state; // Default hold

case (current_state)

S_IDLE: next_state = i_bit ? S_1    : S_IDLE;

S_1:    next_state = i_bit ? S_1    : S_10;

S_10:   next_state = i_bit ? S_101  : S_IDLE;

S_101:  next_state = i_bit ? S_1011 : S_10;

S_1011: next_state = i_bit ? S_1    : S_10;

default: next_state = S_IDLE;

endcase

end

// Combinational Block: Output logic

// Match asserts only when in S_1011 state

assign o_match = (current_state == S_1011);

endmodule

## MODULE 2: Traffic Light Controller

### 2.1 Overview

This FSM module consists of a main road and side road. We need to properly design this so no accident could happen. Each state has a timer until that timer ends it remains in that state and then moves to the next state. The side state also changes with respect to the main state.

### 2.2 Key Features

- Deterministic state rotation (Green → Yellow → Red).

- Interfaces with an external timer module for configurable state durations.

- Enforces a safe all-red state upon reset.

### 2.3 Interface Table

| Port Name | Direction | Width | Clock Domain | Active Level | Description |
| --- | --- | --- | --- | --- | --- |
| clk | Input | 1 | N/A | Rising Edge | Main system clock. |
| rst_n | Input | 1 | Asynchronous | Active-Low | Asynchronous reset. Forces lights to RED in all directions. |
| i_timer_done | Input | 1 | clk | Active-High | Asserted by the timer when the current state duration has expired. |
| o_start_timer | Output | 1 | clk | Active-High | Pulses HIGH for one cycle to restart the timer upon entering a new state. |
| o_main_light | Output | [1:0] | clk | N/A | Main road light status. (00: Red, 01: Green, 10: Yellow). |
| o_side_light | Output | [1:0] | clk | N/A | Side road light status. (00: Red, 01: Green, 10: Yellow). |

### 2.4 Microarchitecture & FSM Specification

State Encoding Table:

| State Name | Encoding ([1:0]) | Description |
| --- | --- | --- |
| S_MAIN_GREEN | 2'b00 | Main road is Green, Side road is Red. |
| S_MAIN_YELLOW | 2'b01 | Main road is Yellow, Side road is Red. |
| S_SIDE_GREEN | 2'b10 | Main road is Red, Side road is Green. |
| S_SIDE_YELLOW | 2'b11 | Main road is Red, Side road is Yellow. |

(Note: Safe Red/Red is handled dynamically during reset before entering S_MAIN_GREEN).

ASCII State Diagram:Plaintext

2.5 Timing Diagrams

- When i_timer_done is sampled HIGH, the state changes on the next rising clock edge.

- o_start_timer pulses HIGH combinationally alongside the state transition to restart the external countdown.

### 2.6 Corner Cases

| Scenario | Expected Behaviour | How it is Handled in RTL |
| --- | --- | --- |
| Reset during Yellow | Must not jump straight to Green. Must initialize in a safe Red state before starting the Main Green timer. | rst_n forces default outputs directly to 2'b00 (Red) before the FSM begins normal operation. |
| Timer set to 0 | If the timer expires instantly, the FSM might flicker. A minimum state time must be enforced. | External timer (or FSM logic) prevents i_timer_done from asserting if elapsed time is 0. |

### 2.7 RTL Code (Revised)

## module traffic_light (

## input  logic clk,

## input  logic rst_n,

## input  logic i_timer_done,

## output logic o_start_timer,

## output logic [1:0] o_main_light,

## output logic [1:0] o_side_light

## );

## // Light Color Encodings

## localparam logic [1:0] RED    = 2'b00;

## localparam logic [1:0] GREEN  = 2'b01;

## localparam logic [1:0] YELLOW = 2'b10;

## // State Encoding

## typedef enum logic [1:0] {

## S_MAIN_GREEN  = 2'b00,

## S_MAIN_YELLOW = 2'b01,

## S_SIDE_GREEN  = 2'b10,

## S_SIDE_YELLOW = 2'b11

## } state_t;

## state_t current_state, next_state;

## // Sequential Block: State Register

## always_ff @(posedge clk or negedge rst_n) begin

## if (!rst_n) begin

## current_state <= S_MAIN_GREEN; // Default FSM start

## end else if (i_timer_done) begin

## current_state <= next_state;

## end

## end

## // Combinational Block: Next State and Timer Trigger

## always_comb begin

## next_state = current_state;

## o_start_timer = 1'b0;

## if (i_timer_done) begin

## o_start_timer = 1'b1; // Pulse timer reset

## case (current_state)

## S_MAIN_GREEN:  next_state = S_MAIN_YELLOW;

## S_MAIN_YELLOW: next_state = S_SIDE_GREEN;

## S_SIDE_GREEN:  next_state = S_SIDE_YELLOW;

## S_SIDE_YELLOW: next_state = S_MAIN_GREEN;

## default:       next_state = S_MAIN_GREEN;

## endcase

## end

## end

## // Combinational Block: Output Logic (Safe-Red on Reset Override)

## always_comb begin

## if (!rst_n) begin

## o_main_light = RED;

## o_side_light = RED;

## end else begin

## // Default to RED unless explicitly overwritten

## o_main_light = RED;

## o_side_light = RED;

## case (current_state)

## S_MAIN_GREEN:  o_main_light = GREEN;

## S_MAIN_YELLOW: o_main_light = YELLOW;

## S_SIDE_GREEN:  o_side_light = GREEN;

## S_SIDE_YELLOW: o_side_light = YELLOW;

## endcase

## end

## end

## endmodule

## MODULE 3: Synchronous FIFO

### 3.1 Overview

A FIFO is module where whichever data comes first also leaves first. It is commonly used to maintain a synchronisation across various devices of different clock so data doesnt get corrupted. As it is a synchronous FIFO data can be written and read at the same time.

### 3.2 Key Features

- Parameterized DATA_WIDTH and DEPTH.

- Simultaneous Read and Write capabilities on a single clock edge.

- Combinationally evaluated full and empty status flags to prevent data loss.

### 3.3 Interface Table

| Port Name | Direction | Width | Clock Domain | Active Level | Description |
| --- | --- | --- | --- | --- | --- |
| clk | Input | 1 | N/A | Rising Edge | Main system clock. |
| rst_n | Input | 1 | Asynchronous | Active-Low | Asynchronous reset. Clears pointers and count register. |
| i_wren | Input | 1 | clk | Active-High | Write Enable. Pushes i_wdata into memory if !o_full. |
| i_wdata | Input | [31:0] | clk | N/A | Write Data. Payload to be buffered. |
| i_rden | Input | 1 | clk | Active-High | Read Enable. Pops oldest data to o_rdata if !o_empty. |
| o_rdata | Output | [31:0] | clk | N/A | Read Data. Driven on the clock cycle following a valid read. |
| o_full | Output | 1 | clk | Active-High | Full Flag. High when count == DEPTH. |
| o_empty | Output | 1 | clk | Active-High | Empty Flag. High when count == 0. |

### 3.4 Microarchitecture Specification

The datapath consists of a 2D memory array (reg [31:0] mem [0:15]). Two internal registers, wr_ptr and rd_ptr, track the current indices. A count register tracks the total number of valid elements. Pointers automatically wrap back to 0 upon reaching DEPTH-1.

### 3.5 Timing Diagrams

- Write Latency: Data presented on i_wdata with i_wren = 1 is written to memory on the next rising clock edge.

- Read Latency: When i_rden = 1, the data at the current rd_ptr is available on o_rdata on the immediate next clock cycle (latency of 1).

### 3.6 Corner Cases

| Scenario | Expected Behaviour | How it is Handled in RTL |
| --- | --- | --- |
| Write when Full (Overflow) | Write must be ignored. Count and wr_ptr must not increment. | Gated by assign write_valid = i_wren && !o_full. |
| Read when Empty (Underflow) | Read must be ignored. Count and rd_ptr must not decrement. | Gated by assign read_valid = i_rden && !o_empty. |
| Simultaneous R+W when Full | Write accepted. Count remains at DEPTH. Both pointers advance. | Combinational o_full allows i_rden to ungate the write. Count holds steady. |
| Simultaneous R+W when Empty | Read blocked. Write accepted. Count increments to 1. | o_empty blocks read. Treated as Write-Only. |

module sync_fifo #(

parameter DATA_WIDTH = 32,

parameter DEPTH      = 16

) (

input  logic clk,

input  logic rst_n,

input  logic i_wren,

input  logic [DATA_WIDTH-1:0] i_wdata,

input  logic i_rden,

output logic [DATA_WIDTH-1:0] o_rdata,

output logic o_full,

output logic o_empty

);

// Internal parameter for pointer widths based on DEPTH

localparam ADDR_WIDTH = $clog2(DEPTH);

// Memory array and pointers

logic [DATA_WIDTH-1:0] mem [0:DEPTH-1];

logic [ADDR_WIDTH-1:0] wr_ptr;

logic [ADDR_WIDTH-1:0] rd_ptr;

// Count needs to go up to DEPTH (e.g., 0 to 16 requires 5 bits)

logic [$clog2(DEPTH+1)-1:0] count;

logic write_valid;

logic read_valid;

// Combinational flags

assign o_full  = (count == DEPTH);

assign o_empty = (count == 0);

// Read/Write Validation Logic

// Corner Case: If full, write is only valid if a read is simultaneously freeing a spot

assign read_valid  = i_rden && !o_empty;

assign write_valid = i_wren && (!o_full || i_rden);

// Sequential Block: Counters and Pointers

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

wr_ptr <= '0;

rd_ptr <= '0;

count  <= '0;

end else begin

// Count Update Logic

case ({write_valid, read_valid})

2'b10: count <= count + 1; // Write only

2'b01: count <= count - 1; // Read only

2'b11: count <= count;     // Simultaneous R/W: Count holds steady

2'b00: count <= count;     // No operation

endcase

// Write Pointer Update (with wrap-around)

if (write_valid) begin

wr_ptr <= (wr_ptr == DEPTH - 1) ? '0 : wr_ptr + 1;

end

// Read Pointer Update (with wrap-around)

if (read_valid) begin

rd_ptr <= (rd_ptr == DEPTH - 1) ? '0 : rd_ptr + 1;

end

end

end

// Sequential Block: Memory Write

always_ff @(posedge clk) begin

if (write_valid) begin

mem[wr_ptr] <= i_wdata;

end

end

// Sequential Block: Memory Read (1 cycle latency)

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

o_rdata <= '0;

end else if (read_valid) begin

o_rdata <= mem[rd_ptr];

end

end

endmodule